AAPKA TUITION CENTRE
GHAZIABAD

INTERNAL PHASE TEST – PHASE 1

PHYSICS

Maximum Marks: 60 						      Time allowed: 2 hours

General Instructions:

- All questions are compulsory.

- Do the paper in a very neat and clearly readable handwriting.

- This paper consists of three sections. The individual section weight age is as follows:

Section A		Q. No. 1 – 8			    	2 marks for each question

Section B		Q. No. 9 – 16 			4 Marks for each question

Section C		Q No. 17 – 18			6 marks for each question

- Do all rough work in the fair copy itself. Please don’t ask for a separate sheet for calculations or any other rough work.

- You may use the useful data and constants for calculations given on Page 2.

__________________________________________________________________________________________________

Useful data:

SECTION – (A)

Each question carries 2 marks

- Name and define the role of the three parts of a communication system?

- Give a complete schematic representation of remote sensing. Give its one application.

- What is AM? Why is AM more susceptible to noise than FM?

- Differentiate between charge and mass.

- Mention two important properties of electric lines of force. Sketch them for an isolated positive point charge.

- (a) An isolated metallic conductor is positively charged. What changes, if any, come in its mass?

(b) What is the direction of electric field at a point on the equatorial line of an electric dipole?

- A 900 pF capacitor is charged by a 100 V battery. How much electrostatic energy is stored by the capacitor?

- In a medium, the force of attraction between two point charges, distance d apart, is F. What distance apart should these be kept in the same medium so that the force between them becomes (i) 3F   (ii) .

SECTION – (B)

Each question carries 4 marks

- (a) What is the full form of LASER?

(b) Write three advantages of using LASER over LED as an optical source?

- Why is modulation necessary in a communication system?

- State the principle of an optical fibre. Explain with a diagram.

An optical fibre is made from core glass of refractive index 1.62 and cladding of refractive index 1.53. Calculate the critical angle and angle of acceptance.

- Derive an expression for the distance upto which the TV signals can be directly received from a TV tower of height h.

A TV tower has a height of 100 m. How much population is covered by the TV broadcast if the average population density around the tower is 1000 /km2. (Radius of Earth = 6400 km)

- Draw a well labeled diagram of a Van – de – graffe generator. State its working principle and also give its two limitations.

- Show that the work done in rotating an electric dipole of dipole moment p in a uniform electric field E by an angle  from the equilibrium position is given by .

- For a parallel plate capacitor, prove that the total energy stored in the capacitor is and hence derive the expression for the energy density of the capacitor.

- A dipole consists of two charges , separated by a distance of . It is placed in a uniform electric field of . Find:

- the electric dipole moment of the dipole

- The maximum torque exerted on the dipole by the electric field.

- The potential energy of the dipole in the equilibrium position,

- The work necessary to rotate the dipole by 180° from the equilibrium position.

SECTION – (C)

Each question carries 6 marks

- Derive an expression of the electric field at a point due to a uniformly charged spherical shell using gauss law and present the result graphically.

A regular hexagon of side 10 cm has a charge  at each of its vertices. Calculate the potential at the centre.

- (a) A particle of mass m and carrying charge – q1 starts moving around a fixed charge + q2 along a circular path of radius r. Prove that the period of revolution is .

(b) Parallel plate capacitor is filled with two dielectrics as shown in the following diagram. Calculate the resultant capacitance?